Wipro Service Assurance

Standard of Procedure

for

Mass Port Recovery

Network-Data/30-Mar-18/Process

Table of contents

1.	Purpose	3

2.	Business as Usual (BAU) Change	3

2.1 Port Recovery	3

3.	Roles & Responsibility	3

4.	Environment	3

5.	Definitions & Acronyms	3

6.	References	3

7.	Implementation Steps	3

8.	Amendment History	4

# Purpose

The Purpose of this Steps of Procedure (SOP) is to:

- Identify the steps for implementation of BAU Changes for Port recovery.

# Business as Usual (BAU) Change

- 2.1 Port Recovery

This activity is useful to recover the un-used or not required ports on the switches.

The following steps provide a high-level overview that outlines Port Recovery request to completion.

Port Recovery:

- A service request is opened with the YYZ Service Desk for Port Recovery on the switch required.

- Counters will be cleared on the switches to identify the long time un-used ports.

- Ports will be recovered from the switches after identifying the ports with no counter increments for 4 weeks and above.

- Task will be assigned to B&M SP team to recover the Patching in the field if any.

- Service request is updated with status and closed.

# Roles & Responsibility

Network Team – Implementing the changes on network switches

B&M SP Team – Recovery of patching in the field.

# Environment

environment is comprised of Cisco equipment’s.

# Definitions & Acronyms

# References

NA

# Implementation Steps

- RDP(mstsc) to Wipro Jump box xxx.xx.xxx/xxx.xxx.xxx

- RDP (mstsc) to Network Jump box 192.168.3.24

- Login to the switch for which the port needs to be recovered and clear the counters with below command for all the interfaces of the switch.

#Clear counters

- Wait period of 4 weeks to check if there is any increment in the counters of the switch interfaces which are in notconnect state.

- After 4 weeks identify ports which are in down/down state and has no increment in counters in the past 4 weeks, use below command to identify such interfaces.

#show interfaces | i notconnect|Last i

e.g.

Note: Interfaces with no increment in counters are highlighted below.

T3-G140-3-81#sh interfaces | i notconnect|Last i

Last input never, output 00:00:30, output hang never

Last input 00:00:21, output 00:00:44, output hang never

Last input never, output 00:00:17, output hang never

GigabitEthernet1/4 is down, line protocol is down (notconnect)

Last input 31w1d, output 17w6d, output hang never

GigabitEthernet1/5 is down, line protocol is down (notconnect)

Last input never, output never, output hang never

Last input never, output 00:00:19, output hang never

Last input never, output never, output hang never

GigabitEthernet1/8 is down, line protocol is down (notconnect)

Last input 30w0d, output 30w0d, output hang never

GigabitEthernet1/9 is down, line protocol is down (notconnect)

Last input 31w3d, output 31w3d, output hang never

Last input never, output 00:00:20, output hang never

Last input 00:00:15, output 00:00:31, output hang never

Last input never, output 00:00:47, output hang never

Last input 00:00:03, output 00:00:03, output hang never

GigabitEthernet1/14 is down, line protocol is down (notconnect)

Last input 2w6d, output 2w6d, output hang never

Last input never, output 00:00:28, output hang never

Last input never, output 00:00:52, output hang never

GigabitEthernet1/17 is down, line protocol is down (notconnect)

Last input 50w6d, output 21w3d, output hang never

GigabitEthernet1/18 is down, line protocol is down (notconnect)

Last input never, output 1y14w, output hang never

- Once interfaces identified as highlighted above, use below steps to recover those interfaces.

Login to the switch and enter below commands.

#Config t

#Default Interface range (optional) <Port number>

#Interface range (optional) <number>

#Shut

#Description <Recovered_Service Request number>

#Exit

#Exit

e.g.

T3-L230-3-95#conf t

T3-L230-3-95(config)#default interface range Fa2/3 , Fa2/20 , Fa2/25 - 26 , Fa2/32 - 34

T3-L230-3-95(config)#interface range Fa2/3 , Fa2/20 , Fa2/25 - 26 , Fa2/32 - 34

T3-L230-3-95(config-if-range)#shut

T3-L230-3-95(config-if-range)#description Recovered_SR1474544

T3-L230-3-95(config-if-range)#exit

T3-L230-3-95(config)#exit

T3-L230-3-95#

- Once recovered assign task for B&M SP Team to recover patching in the field.

- Once Patching recovered, update central Port availability database and close the ticket.

# Amendment History

| Version | Author(s) | Approved By | Nature of Change |
| --- | --- | --- | --- |
| 0.1 | Kiran Patel |  | Creation & Template Update 30/03/18 |
| 1.0 | Yogesh Rajguru |  | Reviewed & approved |

# Thank You